Company Campus Network

Network Design & Implementation Report

using Cisco Packet Tracer

# 1. Overview

This report documents the design of a multi-floor company local area network (LAN) built and simulated in Cisco Packet Tracer. The network spans a Ground Floor, First Floor, Second Floor, and a dedicated Server Room, connected through a central router and distribution-layer switching. Each department or functional zone is isolated on its own VLAN with its own IP subnet and DHCP-assigned addressing, enabling logical segmentation, controlled inter-VLAN routing, and simplified administration.

# 2. Network Topology

The overall physical and logical layout of the network is shown below.

## 2.1 Core Devices

- MAIN ROUTER — central router providing inter-VLAN routing and uplink to the Server Room, Ground Floor, First Floor, and Second Floor distribution switches.

- A multilayer/distribution switch on the First Floor handles routing and switching for HR, Sales, and Marketing VLANs.

- A distribution switch on the Second Floor connects Developer workstations, the IT area, the Office, and the Meeting Room wireless access point.

- A Ground Floor switch connects Reception and a wireless access point serving Guest Wi-Fi clients.

- The Server Room hosts an Application Server and a DHCP Server on their own VLAN, uplinked directly to the main router.

# 3. VLAN & IP Addressing Scheme

Each zone is assigned a dedicated /24 subnet with the router (or multilayer switch, where applicable) acting as the default gateway for that VLAN.

| Department / Zone | VLAN Subnet | Default Gateway | Floor / Location |
| --- | --- | --- | --- |
| Guest Wi-Fi | 192.168.1.0/24 | 192.168.1.1 | Ground Floor |
| Reception | 192.168.2.0/24 | 192.168.2.1 | Ground Floor |
| HR | 192.168.3.0/24 | 192.168.3.1 | First Floor |
| Sales | 192.168.4.0/24 | 192.168.4.1 | First Floor |
| Marketing | 192.168.5.0/24 | 192.168.5.1 | First Floor |
| Developers | 192.168.6.0/24 | 192.168.6.1 | Second Floor |
| IT | 192.168.7.0/24 | 192.168.7.1 | Second Floor |
| Meeting Room | 192.168.8.0/24 | 192.168.8.1 | Second Floor |
| Server Farm | 192.168.9.0/24 | 192.168.9.1 | Server Room |

# 4. Floor-Wise Breakdown

## 4.1 Server Room

Connects directly to the main router, outside the departmental switching layer.

| Device | Role |
| --- | --- |
| Server-PT (Application Server) | Hosts company application services |
| Server-PT (DHCP Server) | Issues DHCP leases to network clients |
| PC-PT (office) | Which have access of entire network |

## 4.2 Ground Floor

Serves Reception staff and Guest Wi-Fi users via an access switch and a wireless access point.

| Device | Role |
| --- | --- |
| Reception PC + Printer | Front-desk workstation and shared printer, on the Reception VLAN |
| Wireless Access Point | Provides Guest Wi-Fi to visitor laptops (Guest VLAN) |
| Guest Laptops (x4) | Visitor devices connected wirelessly to the Guest VLAN |

## 4.3 First Floor

A multilayer switch distributes traffic to three department groups, each wired to its own access switch segment.

| Department | Endpoints |
| --- | --- |
| HR | 6 PCs (HR-1 to HR-6) + shared printer |
| Sales | 6 PCs (Sales-1 to Sales-6) + shared printer |
| Marketing | 6 PCs (Marketing-1 to Marketing-6) + shared printer |

## 4.4 Second Floor

The largest segment, hosting the Developer team, IT support, a private Office, and a Meeting Room with wireless access.

| Zone | Endpoints |
| --- | --- |
| Developers | Multiple developer workstations (Developer-1 through Developer-10) on the Developer VLAN |
| IT | IT support PCs and a shared printer on the IT VLAN |
| Office | A dedicated office laptop |
| Meeting Room | Wireless access point serving 4 laptops for meetings, on the Meeting VLAN |

# 5. Design Notes

- Each VLAN is scoped to its own /24 network with the gateway address ending in .1, keeping addressing consistent and easy to remember across departments.

- Wireless zones (Guest Wi-Fi and Meeting Room) are kept on separate VLANs from wired department traffic to limit broadcast domains and improve security isolation.

- DHCP services are centralized in the Server Room so all VLANs receive address leases from a single, dedicated server.

- The topology follows a hierarchical design: a core router at the top, a multilayer switch for inter-VLAN routing on busier floors, and access-layer switches/access points closest to end devices.

# 6. Conclusion

The simulated network provides logical separation between departments, guests, servers, and shared infrastructure, while remaining centrally routed and manageable from a single main router. This design can be extended with additional floors, VLANs, or redundancy (e.g., a second uplink or router) as the organization grows.